STANDARD OPERATING PROCEDURE FOR SHEARING

- Relevant PPE’s such as hand gloves, safety helmets, and safety shoes are to be used.

- Switch On the machine shear blade up and down checking

- Check the grade and thickness of the material.

- Check for the safety gate provided at the entrance of the shear cut tool (blade).

- Raw material coil is placed on the mandrel with the help of the EOT crane and the mandrel is expanded.

- Before cutting the binding strip ensure the Snubber roll is in contact with the uppermost sheet of the coil or the end of the coil.

- After removing the binding strip using gloves feed the sheet to the shear cut tool through loop pit table.

- After feeding the coil in the shear cut tool part , if there are any uneven edges at the face of the coil then it has to be cut using shearing blade  by pressing the foot lever.

- Raw material is feeded to the shearing blade and cut the material as per required size.

- Collect the job pieces stack in storage area

- Scrap is collected and disposed in scrap bin

- Clean the surface with brush to remove the dust and ensure surface is rust free

SOP NO.: PIL/CRFS-PD-SOP-03 REV.00 DATE: 01.03.2022.