STANDARD OPERATING PROCEDURE OF PRESS BRAKE

- Relevant PPE’s such as hand gloves, safety helmets, and safety shoes are to be used.

- Identify profile sheets with the help of the route card.

- Keep the required material at the press with the help of EOT crane

- Before starting the press check for loose of any clamps of the tool.

- Select the Die and Punch tool according to required shape of the job.

- Check for working of press photo safety guards.

- Check for emergency stopper button working.

- Check for two hand push button operation.

- Check for an abnormal noise in the press.

- Load the component on locater properly.

- Set the blank on the blank holder and touch to the stopper pins on the lower die.

- Operate the machine from control desk and press both the both push buttons at a same time.

- If all parameters are “ok” then start the production.

- Note down the details in production report.

- After completion of the production switch off the machine.

SOP NO.: PIL/CRFS-PD-SOP-04 REV.00 DATE: 01.03.2022.