STANDARD OPERATING PROCEDURE FOR HR SLITTING OPERATION

BEFORE HR SLITTING OPERATION

- Relevant personal protective equipment to be used by operator

- As per the program identify the coil number / width and thickness at HR coil storage yard.

- Assist crane operator in attaching ‘C’ hook to coil, while coil shifting from RM storage to coil car pay off and keep it on coil lifting saddle and make correct position with cones.

- Make setting at slitter arbor, underarm and over arm on coiler drum by seeing the program (as per the given plan) by wearing safety gloves and following safety rules. The setting to be checked with Vernier for correct width.

- Switch on the hydraulic power pack

- Tight the eye of the coil with cones.

- Lower the coil car. Lower the snubber roll with a position of coil end / wrap end at least 0.5meter away from snubber roll towards slit.

- Cut the binding strap with scissor.

- Ensure proper inspection and working condition of safety sensors and safety guards before line start.

DURING HR SLITTING OPERATION

- With the help of snubber roll operation, adjust the guides with respect to the size setting made, feed the sheet through pinch roll after the leveler roll for flat lead end, then pass through shear unit if any holes / cracks or fish tail at the leading end of the coil.

- Release the snubber roll, feed the sheet up to slitting arbor with leveler cum pinch roll operation. When sheet comes to arbor adjust the gap by lowering the arbor till the sheet slit.

- Jog the slitter by lifting pinch roll & leveler for slitting coil, adjust the guides and the arbor for good slitting i.e., no damage on the slitting coil.

- Check the width of slit coils with steel rule / measuring tape and confirm it to the program.

- Jog / Operate the slitter till the slit parts goes to coiler drum.

- Insert the slit parts into gripper and expand the mandrel.

- Keep the over arm down. Jog / inch the coiler drum up to one wrap would as cover.

- Continue the line run until the last wrap ends on the payoff cones, and observe the following things:

- Avoid for overlap of the slit parts.

- Slit the coils according to program whether in full part or parts to be made

AFTER HR SLITTING OPERATION

- After releasing the outer wrap of the coil at cones, operate coiler on jog mode from line run up to wrap end comes under over arm to facilitate the binding of slit coils.

- Binding to be made for all slit parts and lift the over arm.

- Position the mandrel such that the strip in gripper should match to center of the turn style arm and collapse it.

- Strip out the slit coils onto the coil saddle. Give identification with stickers.

- Place the slit coils in storage area for transfer to SCP as required.

- Full hand sleeves should be used during scrap removal from scrap winder/loose scrap. Write the production report.

SOP NO.: PIL/11MFG/PM/06 REV.00 DATE: 01.10.2021.