STANDARD OPERATING PROCEDURE FOR 4 HI-MILL

4 HI-Mill OPERATION

- Relevant personal protective equipment to be used by operator.

- Go through rolling plan

- Check –up whether proper work rolls are available.

- Identify coil with coil /sticker and check width and gauge according to plan.

- Transfer the selected coil from awaiting rolling area with E0T crane to pay off reel skid.

- Load the coil from skid to pay – off reel with the help of coil car.

- Centre the coil and expand the pay-off mandrel

- Lower the hold-down (snubber) roll

- Cut the binding strap and feed the head end of the coil manually up to pinch roll by rotating pay-off reel in forward direction.

- Lower the pinch roll to hold the head end of the coil and lift the hold down (Snubber) roll

- Jog the pinch roll and bring the head end up to shear.

- In case the head is having uneven or bend edges shear the bad portion with the help of shear/gas cutting

- Ensure that roll gap between work –roll is more than the strip initial thickness

- Check the coil width, thickness at both edges of the width and enter the details in the    4 Hi- Mill Production Report.

- Lower the bridging apron (if applicable) and fed the strip up to entry side deflector roll

- Open the guide more than the strip width and set the top and bottom wooden drag-board

- Operator should arrange wood box in milling house before entry of the sheet and check the supply of the coolant in the housing mill

- By Jogging the “forward” main drive, feed the strip through drag -board and work roll gap with help of pinch roll or pay-off reel

- Tight the guide according to strip width with the help of wooden drag-board and down drag-board (Pneumatics pressure)

- Give the load on required to the first pass load on the end of strip and switch on the pay-off tension

- the strip mover rolls to delivery tension reel operate

- the mill in thread mode till the strip moves from roll bite to delivery tension reel

- Ensure levelling is proper by adjusting individual screw.

- Feed the strip to the delivery mandrel gripper and “Expand” the mandrel. Ensuring head-end is properly inserted inside the gripper.

- Rotate the delivery, mandrel jog in Jog mode and Main drive so that around one wrap of coil is wound on the mandrel.

- Stop the jogging and switch on delivery tension.

- Feed pass schedule in PLC Programme

- Select first pass from delivery side panel board

- Apply required load. Start the coolant spray and start the mill.

- Check thickness by measuring with Micro meter, compute the difference observed during physical checking with non-contact gauge measuring unit.

- The rolling should continue as per plan schedule till we get required thickness’s

- Put a binding strap around the coil, give identification with coil card/ sticker and sent to next work center.

- Quality inspection with hand instrument should not carried out while machine running

- Write the production report.

SOP NO.: PIL/11MFG/PM/10 REV.00 DATE: 01.10.2021.