STANDARD OPERATING PROCEDURE FOR SKIN PASS

- Relevant personal protective equipment to be used by operator

- Check the rolls availability and choose the rolls (bright /fine matt /matt) as per annexure.

- Operator should place the rolls in skin pass mill carefully as per annexure.

- Identify the coil either with sticker or by identification written by marker.

- Operator has to guide EOT crane operator in placing identified coil to the pay off skid

- Before lifting the coil if any inner diameter wraps are present then it should be gas cut.

- After placing the coil in pay off skid with the help of the coil car the coil is placed on the pay off mandrel by operating coil car in/out, up/down.

- Centre the coil and expand the pay off mandrel.

- Lower the snubber roll and contact the roll to the coil.

- Position the peeler table at control desk and manually cut the binding strip using the cutter tool.

- Feed the head end of the coil to the shear by rotating the pay off reel in forward direction.

- Uneven edge of the lead end of coil is cut on shearing.

- To ensure the straight movement of lead end to the mill house guide rolls are placed.

- Ensure the roll gap between the work roll is more than the strip initial thickness.

- And operator should give the load such that it will improve strip flatness.

- Ensure the flatness of the strip with positive and negative bending load, and level the strip by squeezing if necessary.

- After shearing, pass the strip up to coiler via deflector roll, oiler and forward stretcher to lower thickness and into mandrel to higher thickness.

- Wait for Q.C clearance for skin pass operation.

- Then pass the strip through deflectors and oil rollers where lubricants are done all over the surface of the coil.

- Feed the strip deliver mandrel and expand the mandrel.

- Ensure the strip is properly inserted inside the grooves of the gripper mandrel.

- After the completion of recoiling the binding strip is placed and sticker/marker is attached to the coil.

- Coil made out by supporting coil car and mandrel collapse.

- Write the production report.

SOP NO.: PIL/11MFG/PM/14 REV.00 DATE: 1.10.2021.