STANDARD OPERATING PROCEDURE FOR CRS-5 SLITTING

- Relevant personal protective equipment to be used by operator

- Identify the coil with sticker

- Transfer the coil by means of EOT crane to uncoiled pedestal/saddle

- Stock of rust preventive oil in storage tank to be checked and it should be filled as per level.

- For above work use pathway and ladder provided at machine.

- Ensure / check availability of fire extinguisher & water during the staring of soot cleaning process.

- Put the uncoiler mandrel in collapsed position and load the coil to uncoiled mandrel with the help of coil car.

- Expand the uncoiler mandrel after placing the coil at centre of the mandrel

- Snubber is rolled down and the binding strip is cut using binding cutter tool.

- Lower the pinch roll and run it in forward direction, operator should not stand front of the coil

- Cut the unrolled portion length and the sample is given to quality inspector for properties checking

- Make setting at slitter arbor and pass line or underarm setting at drag board if necessary at over arm on recoiler.

- Pass the sheet through entry guide roll  unit to slitter arbor

- From slitter shearing scrap on both edges is wound on scrap winder, scrap removal with full hand sleeves

- Slit parts are taken to the coiler through drag board unit, pinch rolls, exit deflector roll and coiler roller.

- Insert the slit parts in to the coiler gripper and expand the mandrel

- Check the Initial winding at recoiler before putting machine run mode

- Adjust drag board and brake pressure

- Avoid overlap if it is multiple setting.

- Make ready the Rust Preventive oil in tank then start the line as clearance for quality check about properties

- Soot cleaning system is introduced to clean surface of the strip with kerosene wiping (if required), In each shift cleaning of floor / house keeping to be done

- Coil is cut and recoiled after slitting as per customer outer diameter

- Slit coil is stripped out to turn style

- Line is stopped in between for visual inspection, thickness checking and properties checking

- Write the production report

Don’t DOs:

- During running or in idle condition operator should not cross through equipment to reach drive side.

- Don’t use other than hammer during setting change for unlock / lock of nuts at upper arm / under arm and at slitting head.

SOP NO.: PIL/11MFG/PM/15 REV.00 DATE: 01.10.2021.