STANDARD OPERATING PROCEDURE FOR PACKING

WORK INSTRUCTIONS FOR OPERATORS

- Based on the awaiting packing coils/pallets, mode of packing is verified from work order.

COIL FORM PACKING

NACKED PACKING:

- Ensure the identification stickers

- Make cross binding with 2 or 3 and tag the coil

- Weight the coil with the help of EOT crane and enter in packing report

- Keep the coil in FG Area with proper supports i.e to arrest rolling/falling

POLY CRAFT PAPER/HESSAIN:

- Ensure the identification stickers

- Make cross binding with 2 or 3 Nos

- PCP/Hessian wrapping through eye of the coil. By rolling if it is >150mm width and an turn style if it is <150mm

- Radial binding (more than 50mm) cross bindings and tag the coil.

- Weight the coil with the help of EOT crane and  enter it in packing report

- Keep the coil in FG Area with proper supports i.e to arrest rolling/falling

- With polythene wrapping should not lift with C-Hook.

POLY CRAFT PAPER / HESSAIN, POLYTHENE

- Ensure the identification stickers.

- Make cross binding with 2 or 3Nos

- Polythene wrapping through eye of the coil

- By rolling on floor if it is >150mm width and on turning if it is <150mm

- PCP/Hessian wrapping through eye of the coil.

- Radial binding, cross binding & tag it

- Weigh the coil with the help of EOT crane and enter it in packing report

- Above 500kg coil put wooden blocks side by with cross binding

- Three or four coils of the same size may be joined together with binding strap for easy handling

- Weigh the coil with the help of EOT crane and enter it in packing report.

- Keep the coil in FG Area with proper supports i.e to arrest rolling/falling

- Stack the coils in narrow width stands only				           Page 1/3

PCP/Hessian, Polythene & Wooden Crate:

- Ensure identification stickers

- Make cross binding with 2 or 3 No

- Radial ;binding, cross  binding & tag it

- Weigh the coil with the help of EOT crane and enter it in packing report

- Keep the required wooden crate. Cover the packed coils with PCP/Hessian and bind it,

- Close the box with wooden reaper and tag it.

- Keep the wooden crate in FG Area

PCP/Hessian, polythene and steel cover:

- Ensure identification stickers

- Polythene wrapping through eye of the coil

- Make ready the cut CR sheets bind it to cover outlet and inner edges of coil

- Wrap the PCP/Hessian over metallic shielding

- Keep the wooden blocks at side edges and bind it

- Make cross and radial binding with the tags ID &OD

- Weigh the coil and enter it in packing report

- Keep it in FG Area with proper support such that it should not roll or fallen on ground

Skid Packing:

- Ensure identification stickers. Make cross binding with 2 or 3 Nos

- Radial binding cross binding & tag it

- Weigh the coil with the help of EOT crane and enter it in packing report

- Bring the wooden pallet, keep the weighed coils, on it bind it and tag it (for extra precautionary PCP may be used for whole coverage

- Keep it in FG Area with proper support to avoid damage to wooden pallet and material

Page 2/3

CUT TO LENGTH PACKING:

Poly Craft Paper /Bituminous Hessian, Polythene With Wooden Pallet

- Ensure identification stickers

- Cover the polythene, PCP/Bituminous Hessian over the sheets properly which is provided at CTL.

- Tight the pallet with straps over PCP/Bituminous Hessian

- Put identifications stickers along with binding strap (one big metallic plate)

- Weigh the Pallet with the help of EOT crane and enter it in packing report.

- Keep the pallet in FG Area.

Polythene & Wooden Pallet:

- Ensure identification stickers

- Cover the whole sheets with polythene properly

- Tight the wooden pallet by binding with the identification stickers (one big Metallic plate)

- Weigh the pallet, enter it in the packing report.

Poly craft paper /bituminous Hessian, Polythene, steel cover and wooden pallet.

- Ensure identification stickers

- Cover the polythene, PCP/BH over the sheet properly

- Tight the pallet with two straps over PCP/BH

- Make ready the CR/GP sheets to the required size and cover the whole pallet with CR/GP sheets

- Tight it binding strap and put identifications stickers along with strap (one big metallic plate)

- Weigh the pallet with EOT crane and enter it in packing  report

SOP NO.: PIL/11MFG/PM/17 REV.00 DATE: 1.10.2021.    Page 3/3