STANDARD PROCEDURE FOR PLASMA CUTTING

- Recommended PPEs to be used.

- Load the programme as per the drawing required dimensions.

- Load the raw material using EOT crane with help of magnetic tong and placed on cutting bed.

- switch it to ON fume extraction system.

- Ensure machine is correctly set up for compressed air, nitrogen and oxygen as required.

- Ensure work return cables make firm contact to provide a good electrical connection.

- Metals coated with or containing materials that emit toxic fumes should not be heated or cut unless coating is removed from the work surface.

- Do not leave the plasma cutter running unattended.

- Switch off the machine and fume extraction (where used).

- Turn off air supply and hang up handpiece, hose and welding cables.

- Cleaning of M/C and bed to be carried out once in a month.

- Add to proper place – check to the E stop operation before start using.

SOP NO.: PIL/55FAB/PROD/PM/D02 REV.00 DATE: 27.08.2021