STANDARD OPERATING PROCEDURE FOR PUNCHING TOOL ASSEMBLY

- Use safety helmet, safety shoes and cotton hand gloves & Goggles.

- Identify proper tool from tool room which is to be loaded on the press.

- Tight the strip, punch and punch plate before lifting the tool.

- Lift the tool from tool room using wire rope to ear rings by EOT crane/forklift.

- Keep the press ram in down position and slide in up position.

- Measure the tool height and adjust the press shut height.

- Keep the tool on press bed with forklift and adjust the slide.

- Clamp the tool bottom bolster and top bolster tightly and ensure there is no play and reduce the load.

- Keep one component in the tool and set the required load.

- While removing the tool ensure, top bolster is on the pillars of the bottom bolster die.