# SAFE OPERATING PROCEDURE FOR SHEARING MACHINE

- Purpose: To provide guidelines for easy process for  Shearing Machine.

- Scope: This procedure is applicable to applicable to  Shearing Machine.

- Responsibility: Production Shift In-charge and operators responsible for this activity.

- Procedure:

  - Select the Raw Material as per the B.O.M.

  - Clean the machine, accessories, hydraulic power pack and working area

  - Check the Sharpness of Shearing Blades by taking " TRIM CUT ".

  - Check the " Clearance " between Shearing Blades.

  - Operate the machine on " NO - LOAD " condition for 3 - 4 strokes.

  - Check the working of Hydraulic strokes.

  - Push the angel inside of the machine and take One Trim Cut for every NEW Plate.

  - After " Trim Cut " put the " MARKING " on angle for required length cutting with the help of measuring tape and mark by pencil / chalk.

  - After " MARKING " of angle match the top side blade edge to that marked portion and then " CUT " the Plate by pressing the Foot Switch.

  - After " First Piece " cutting " CHECK " its size if, found " OK " then only start the production.

  - If there is burr on edges, then adjust the clearance between the blades.

  - After Plate cutting keep the angle lot wise. DO NOT “MIX " the lots.

  - Enter the Part Number, Length and Quantity in Log Book for Angle Machine

  - Complete the shearing operation for all the items.

  - Operator will do " CLIT " on daily basis before starting the machine.

  - To start power pack and main switch.

  - Check the blade sharpness.

  - After cutting angles mark the identification number by white paint metal marker.

  - After cutting the angles shall be kept lot wise.

  - Maintain the " GAP " of Shearing Blades 5% of Angle thickness –

  - Maintain the machine in good working condition. He keeps his working area clean.

  - In case of breakdown of the machine he informs his supervisors who in turn advices the plant maintenance department through the breakdown slip. Use of ear plugs, helmets, safety shoes, hand gloves is compulsory.

  - DON'T handle the angle with bare hand on machine at the time of cutting.

  - NEVER put your hands under the clamps or blades when the machine is " ON ".

SOP NO.: PTC/Fab/PROD/ REV.00