STANDARD OPERATING PROCEDURE (SOP)

for TURNING OPERATION

- Use the Safety Helmet, Safety Shoe, Safety Goggle, and Red Cotton Gloves & Apron.

- Only trained/authorized operator in allowed to operate the machine.

- Work under supervisor/shift in-charge/HOD.

- Inspect the machine and check that the lathe is clean and safety guards are in place.

- Turn the chuck by hand to confirm the work piece will not hit the chuck or tailstock.

- Never leave the chick key in the chuck, remove it and secure it in its holding place.

- Carefully select the correct spindle speed using the feed chart for the specific material being cut.

- Securely mount the cutting tool in the tool post and set its height.

- Use cutting fluid as needed to prevent the work piece from heating up.

- Stop the machine to measure the work piece and adjust the cut depth as required.

- Keep a constant watch on the working area.

- When work piece is ready press the stop button.

- Separate the finished products.

- Don’t use the mobile phones during the work activity.

- Maintain safe distance from the machine in running condition and don’t touch by hand.

- Clean the area, machine, tools after completing of job.

- Use this             type of tool for removal scrap from the turning location don’t use bare hands/ Gloved hands to be removed.