STANDARD OPERATING PROCEDURE for STORAGE OF FLAMMABLE MATERIAL.

Use personnel protective equipment’s such as Safety helmet, safety shoes, cotton gloves.

1. Storage location

- Use designated Location: Store flammable liquids in an approved flammable liquid storage location/cabinet with self-closing doors.

- Do not store on the floor: Keep containers off the floor to prevent spills and damage. Do not block exits, aisles, or emergency equipment.

- Segregate incompatible materials: Store flammable materials such as paints, thinners, and techtyle separately and away from oxidizing agents or ignition sources. LPG cylinders must be stored in a dedicated, well-ventilated area, isolated from all flammable liquids as mentioned above to prevent any hazardous reactions or fire risks,

A. Containers and equipment

- Use approved containers: Store flammable liquids in approved safety cans or Department of Transportation (DOT) approved containers. Use secondary containment, such as a tray or bin, for added spill protection.

- Label clearly: All containers must be clearly labeled with the contents and associated hazards. If a flammable chemical is transferred, the new container must be labeled with the same information.

B. Storage environment

- Control ignition sources: Eliminate all ignition sources from storage areas, including open flames, smoking, pilot lights, sparks, and static electricity. Clearly post "No Open Flames" signs.

- Control temperature: Store materials in a cool, dry, and well-ventilated area, away from direct sunlight and heat.

C. Static electricity control

- Bond and ground during transfer: When transferring flammable liquids between metal containers, electrically interconnect (bond) the containers to equalize electrical potential. Grounding is necessary to dissipate static charges to the earth.

- Transfer safely: Transfer liquids slowly to minimize the generation of static electricity. Use a grounded nozzle extension when filling non-conductive containers.

2. Emergency procedures

A. Spills

- Minor spills: For small, contained spills, use spill kit. Use non-sparking tools for cleanup.

- Major spills: Immediately evacuate the area, notify personnel, and call emergency services. Do not attempt to clean up large or hazardous spills.

B. Fire

- Call emergency services: Use fire extinguisher/hose reel/sand buckets to stop fire and call emergency security, safety and section in charge.

- Use the correct extinguisher: If trained and the fire is small, use a Class B or ABC fire extinguisher

C. Exposure

- Inhalation: Move the affected person to fresh air and seek medical attention.

- Skin or eye contact: Flush the affected area with water for 15 minutes, remove contaminated clothing, and seek medical attention.

7. Waste disposal

- Follow regulations: Do not dispose of flammable waste by pouring it down the drain or putting it in regular trash.

- Label waste: Collect flammable chemical waste in properly labelled, compatible, and sealed containers.

.

SOP NO:  PIL/ETP/SFM/05 REV. NO. 6   DATE: 25.08.2025