STANDARD OPERATING PROCEDURE for

RAW MATERIAL STORAGE FOR HR COILS

- PPE: Relevant PPE’s such as Red helmet, safety shoes & Reflective jacket are used for crane operator and helper used cotton hand gloves, yellow helmet and safety shoes.

Storage facility management

- Store coils securely:

  - Store HR coils properly in the designated coil storage area, ensuring they are placed securely and safely to avoid movement or falling.

  - Lifting equipment/slings capacity: Use 25 percent higher capacity of EOT crane and lifting chains than the lifting material.

  - Authorised Crane Operator: Crane operator should be trained and authorise and before lifting material should ensure the safe condition of capacity of crane, material and lifting chains and hook.

  - Coils or HR coils never store closely to the building walls or machinery, walkways the at least one feet distance.

  - All HR coils should be store without slide or slope to prevent sliding

  - Use wooden spacers between stacked coils to prevent roll of the coil.

  - Provide the wooden stopper for vehicle tyres to prevent movement of vehicle and apply the hand brake of vehicle while loading and unloading time.

  - High width surface coils or plates should be bottom and low width coils or plates comes to top during stacking.

SOP NO: PIL/RMS-SOP-07   REV.01   DATE: 17.10.2025