# SAFE OPERATING PROCEDURE FOR COLLECTION, Handling, STORING    & DISPOSAL OF Oil soaked cotton waste

- Purpose: To provide guidelines for easy process for Collect, handle, store disposal of Oil soaked cotton waste

- Scope: This procedure is applicable to collect, handle, store disposal of oil soaked cotton waste

- Responsibility: In-Charge-Stores, and In-charges of all sectional are responsible to implement the instructions.

- Procedure

- Oil soaked cotton waste will be collected into bins provided at respective work areas. Oil soaked cotton waste will be packed into HDPE bags.

- Respective sectional in-charge will send oil soaked cotton waste bags to stores periodically with weighment slip or written note.

- Stores section in-charge will issue acknowledgement note of receipt material to respective sectional in-charge.

- Respective sectional will keep acknowledgement note given by stores as a proof of receiving material.

- Stores department has already identified customers who are authorized by respective pollution control boards.

- Oil soaked cotton waste will be disposed off for the internal use for pre heating purpose and to the authorized operators only.

- The stores will maintain the necessary documents to meet legal requirements. They are Form - 3, Form – 11, Form - 12, & Form -13.

- Stores personnel will maintain appropriate records for receipts, disposal and stocks. - Store in a metal safety can with a tight-fitting lid - Label the container "Contaminated Waste" - Keep containers away from ignition sources - Store in a designated area with a proper slope and spillage collection pit - Remove from the site as soon as possible - Empty individual containers into a larger drum with a lid at the end of each day - Keep drums and containers securely closed and locked - Hand over to a Hazardous Waste Treatment, Storage, and Disposal Facility (TSDF) - Hand over to an authorized recycler. - Maintain proper fire extinguishing equipment and smoke detectors - If there is a small fire, use dry chemical, CO2, sand, earth, water spray, or regular foam - If there is a large fire, use water spray, fog, or regular foam - Ensure that personnel wear protective gas masks when opening drums - Provide exhaust/suction blowers in the area where drums are opened

SOP NO.: PTC/PROD/ REV.00