STANDARD OPERATING PROCEDURE FOR PUNCHING

- Use Helmet, Gloves, Safety Shoes, Ear plug.

- Shift rolled profiles by EOT crane to the press area.

- Before starting the machines, machine guards / sensors are to be checked thoroughly.

- Punch the holes by using punching tool and the stoppers have to be set according to the drawing dimensions.

- Setting has to be done by using template.

- Check the profile as per the drawing (customer).

- Check the locations of holes die, with tape / fixture.

- First component is to be checked by Quality inspection team, If found OK start the production.

- If found not OK do necessary adjustment.

- At regular intervals profile dimensions should be checked.

- Punching scrap on the bed should be removed, periodically transfer to collecting bin.

- After completing the job, ensure switching off the machine and clean it properly.

SOP NO.: PTC/22CRFS/PROD/PM/D03 REV.00 DATE: 26.08.2021