STANDARD OPERATING PROCEDURE FOR

SPOT WELDING OF LOOSE COMPONENTS OF PROFILES

- Ensure to wear the safety helmet, safety gloves, safety goggles, and safety shoes .

- Shift the profile (i.e. Discharge Electrodes, Collecting Electrodes Etc.,) to the spot welding machine area.

- Clean surface from oil and rust from profile and loose components.

- Set the machine for weld strength as per the chart in case not ok, inform superiors.

- Set the profile into the welding fixture and wherever provided, clamp them.

- If Ok continue to spot weld and check in between for strength.

- If any holes and burns are observed take suitable actions, by ensuring the clearance of the profiles.

- Ensure the pitch of the spot as per drawing.

- Ensure the diameter of the spot as per the drawing.

- Ensure spots are free from holes and hand burning.

SOP NO.: PTC/22CRFS/PROD/PM/D04 REV.00 DATE: 01.06.2025