STANDARD OPERATING PROCEDURE FOR DRILLING

- Use Safety shoe, Safety Helmet, Safety Goggle and Cotton Hand Gloves.

- Workplace free from unwanted material and clean.

- Check that the drill chuck guard is in position.

- Ensure the chuck key (if used) has been removed from the drill chuck.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Follow correct clamping procedures to ensure work is secure.

- Adjust spindle speed to suit drill or cutter diameter.

- Never leave the Drill Press while it is running.

- Before making adjustments or before cleaning swarf accumulations switch off and bring the machine to a complete standstill.

- Feed downwards at a sufficient rate to keep the drill cutting.

- Feed with care as the drill breaks through the underside of the work.

- Don’t use hand gloves while drilling.

- Use a safe working posture (beware of hair and cloth catching).

- Switch off the machine and clean the area.

SOP NO.: PIL/TARA/PROD/02 REV.00 DATE: 15.09.2025