STANDARD OPERATING PROCEDURE (SOP)

for EOT CRANE OPERATION

- Crane operator must use the Red Safety Helmet, Safety Shoe and Red Reflecting Jacket.

- Only trained and authorized crane operators to be deployed for EOT Crane operation as per authorized list.

- Ensure the area is clear of people before operating the crane.

- EOT Crane should be operating by pendant within 1 meter of working area i.e. loading and unloading and lifting and shifting of material.

- Keep a constant watch on the working area.

- Before operating the EOT Crane, Operator must ensure to check the condition of Pendent, Pendent Cable, Hook, Cable Trolley, Brakes, Emergency Switch, Crane SWL, Lifting Weight, Chain, Web Sling, Wire Rope before taking up the operation. If notice any abnormalities “DON’T OPERATE THE CRANE AND IMMEDIATE INFORM TO AREA INCHRGE”. After the clearance from area in-charge, to proceed the crane operation and ensure the abnormalities rectified and start the crane operation by crane operator.

- Only one bundle allowed at a time to lifted under the Safe Working Load.

- During LT operation the crane operator should apply bell/Alarm to alert other persons to move away from the load area.

- Using Lifting hook/beam operator should lock latch to the hook.

- Crane operator ensure that nobody should under the suspended load.

- Locating C hook into coils operator should take help from other operator from the machine guide.

- Don’t use the mobile phones during transporter operation, if found to use the mobile phone the person is punishable.

- If the tube it is more than 3 meter spreader beam should be used for lifting the lot.

SOP NO.: PIL/PRODUCTION/SOP-01 REV.02 DATE: 18.07.2025